Autonomous Traffic Intersection Management for Cross-Traffic Control

Authors

| Name | Matriculation |
| --- | --- |
| Nader Yahia | 2210026 |
| Hassan Sazid | 2220046 |
| Mostafizur Rahman | 2219941 |
| Umer Hayat | 1232270 |

2026

# Chapter 1 – Introduction

## 1.1 Problem Statement

Conventional traffic light systems allocate intersection access using static or semi-adaptive timing strategies. These approaches often cause unnecessary waiting times, reduced throughput, and inefficient utilization of intersection space, particularly under varying traffic conditions. For autonomous and connected vehicles, traffic-light-based control may no longer be the most efficient solution. A more advanced approach is required in which vehicles coordinate their movements through a centralized intersection controller capable of dynamically assigning safe intersection reservations. However, implementing such a system introduces significant challenges related to real-time scheduling, conflict resolution, safety guarantees, communication delays, and embedded-system constraints.

## 1.2 Motivation

Recent advances in autonomous driving, V2X communication, and embedded computing enable the development of intelligent intersection management systems that operate without traditional traffic signals. By replacing stop-and-go traffic control with reservation-based scheduling, vehicles can traverse intersections more efficiently while maintaining safety requirements. The motivation for this project is to investigate how cyber-physical system principles, real-time scheduling algorithms, and embedded-system implementation techniques can be combined to create a safe, efficient, and verifiable smart intersection controller. Furthermore, the project provides an opportunity to evaluate the feasibility of implementing such a system on resource-constrained embedded platforms while preserving deterministic real-time behaviour.

## 1.3 Project Objectives

The primary objective of this project is to design and model an intelligent reservation-based intersection management system for autonomous and connected vehicles.

The specific objectives are:

- Develop a formal traffic model for a four-way urban intersection with two lanes per approach.

- Design a conflict-aware vehicle coordination mechanism.

- Develop a reservation-based scheduling strategy for safe intersection access.

- Minimize vehicle waiting times and improve traffic throughput.

- Ensure collision-free operation through conflict detection and arbitration mechanisms.

- Model the system as a cyber-physical system suitable for real-time execution.

- Prepare the architecture for FreeRTOS-based embedded implementation.

- Establish a foundation for formal verification using UPPAAL.

## 1.4 Research Questions

### RQ1 – Traffic Modelling: How can vehicle arrivals, lane occupancy, and intersection conflicts be modelled accurately to represent traffic behaviour at a smart intersection?

### RQ2 – Traffic Optimization: How can reservation-based scheduling improve traffic flow efficiency, reduce waiting times, and maintain collision-free operation compared to conventional control approaches?

### RQ3 – Embedded Real-Time Implementation: How can the proposed smart intersection controller be structured and implemented as a deterministic real-time embedded system while satisfying timing, safety, and resource constraints?

## 1.6 Scope and Assumptions

This project focuses on a single four-way urban intersection consisting of two lanes per approach. The intersection is controlled by a centralized reservation-based controller and does not use conventional traffic lights.

The following assumptions are adopted:

- Vehicles are autonomous and capable of V2X communication.

- Reliable sensor information is available to the controller.

- Vehicle intentions are known before entering the intersection control zone.

- The intersection controller possesses a global view of the intersection state.

- Communication delays remain within acceptable operational limits.

- Pedestrian traffic and non-autonomous vehicle behaviour are outside the scope of this project.

- The work emphasizes system modelling, scheduling, architecture design, and verification rather than deployment on physical vehicles.

## 1.7 Report Structure

The remainder of this report is organized as follows:

- Chapter 2 presents the requirements engineering process, stakeholder analysis, operational scenarios, and system requirements.

- Chapter 3 introduces the system architecture and modelling activities, including component decomposition, activity diagrams, state machines, and sequence diagrams.

- Chapter 4 discusses traffic modelling, scheduling concepts, and conflict-resolution mechanisms.

- Chapter 5 presents the embedded-system design considerations and real-time implementation strategy.

- Chapter 6 describes verification and validation approaches, including formal modelling considerations.

- Chapter 7 summarizes the results, discusses limitations, and outlines future work.

Chapter 2 – System Requirements Engineering

## 2.1 Stakeholder Analysis

| Layer | Main stakeholders | Role in the system |
| --- | --- | --- |
| Layer 1 Vehicle Agents | Autonomous vehicles; human-driven or partially connected vehicles | Send intent/state data; receive movement permission or guidance; introduce deterministic and stochastic behaviour inputs. |
| Layer 2: Sensing & Perception | V2X communication module; roadside sensors; state fusion unit | Convert raw sensor/V2X information into one global lane-level intersection state. |
| Layer 3: Control Layer | Intersection Control Unit (ICU) | Decision core: conflict detection, conflict graph construction, time-space reservation, and lane-level arbitration. |
| Layer 4: Supervisory Layer | Traffic authority; emergency override system; system operator | Inject policy rules, priority constraints, emergency pre-emption, configuration, and maintenance actions. |
| Layer 5: Environment Constraints | Intersection geometry; multi-lane approaches; kinematic limits; V2X latency | Define physical and communication bounds that restrict all scheduling decisions. |

| Entity | Role in the system |
| --- | --- |
| Autonomous vehicles | Provide trajectory intent and vehicle state; cooperate with ICU scheduling. |
| Human-driven / partially connected vehicles | Represent uncertain behavior; may require conservative safety margins and inferred state estimation. |
| Emergency vehicles | Require highest-priority preemption through the intersection when active. |
| Intersection Control Unit / Edge Server | Central scheduler that calculates safe crossing order, reservations, and permissions. |
| Roadside Units (RSUs) | Host communication devices and may support local edge processing near the intersection. |
| Roadside sensors | Cameras, radar, or loop detectors used to observe lane occupancy and detect obstacles. |
| V2X communication network | Transfers vehicle state, intent, scheduling messages, and priority requests. |
| Traffic authority / city operator | Defines rules, legal constraints, priority policies, and deployment requirements. |
| System operator / maintenance team | Maintains sensors, communication modules, and computing infrastructure. |
| Environment constraints | Geometry, multi-lane structure, vehicle dynamics, and communication latency limits. |

## 2.2 Operational Scenarios

| Scenario | Idea | Operation |
| --- | --- | --- |
| S1 Normal Operation | Normal vehicles are handled using FCFS and reservation scheduling. | Vehicle request -> FCFS queue -> conflict check -> reservation -> permission |
| S2 Non-Conflicting Movement | Safe vehicles can cross together when their paths do not conflict. | No conflict detected -> same/overlapping slot -> allow crossing |
| S3 Conflicting Movement | Conflicting vehicles must be separated by time to avoid collision. | Conflict detected -> arbitration -> separate time slots -> safe crossing |
| S4 Emergency Vehicle | Emergency vehicles get highest priority while safety is still checked. | Emergency request -> override FCFS -> normal vehicles stop/wait -> priority permission |
| S5 Sensor/V2X Failure | If sensor or V2X data is unreliable, the system switches to safe mode. | Missing/delayed data -> uncertain state -> stop/wait command -> safe mode |

## 2.3 Functional Requirements

| ID | Requirement | Description | Input | Output |
| --- | --- | --- | --- | --- |
| FR-1 | Vehicle State Acquisition | System shall collect real-time vehicle state data | V2X, sensors | Structured lane-wise vehicle state |
| FR-2 | Lane Monitoring | System shall maintain lane occupancy and queue state | Vehicle arrivals | Lane state model |
| FR-3 | Trajectory Interpretation | System shall map vehicle intent to intersection paths | Vehicle intent | Conflict zone path mapping |
| FR-4 | Conflict Detection | System shall detect spatial-temporal conflicts | Trajectories | Conflict graph |
| FR-5 | Time-Space Reservation | System shall assign non-overlapping intersection slots | Conflict graph | Schedule plan |
| FR-6 | Movement Authorization | System shall issue movement permissions | Schedule | Vehicle commands |
| FR-7 | Scheduling Update | System shall update schedule in real time | New arrivals | Updated schedule |
| FR-8 | State Maintenance | System shall maintain global intersection state | All inputs | System state model |
| FR-9 | Deadlock Prevention (Functional) | System shall avoid full intersection blocking | Lane states | Safe schedule |
| FR-10 | Emergency Override Handling | System shall prioritize emergency vehicles | Emergency input | Preempted schedule |
| FR-11 | Fault Tolerance | System must degrade safely under failures | Safe fallback mode |  |

## 2.4 Non-Functional Requirements

| ID | Requirement | Description | Constraint |
| --- | --- | --- | --- |
| NFR-1 | Real-Time Responsiveness | System must respond to events within bounded time | ≤ 50 ms latency |
| NFR-2 | Determinism | Same input must produce identical output | 100% deterministic |
| NFR-3 | Scalability | System must handle all lanes under peak load | 8 lanes active |
| NFR-4 | Availability | System must operate continuously | ≥ 99.9% uptime |
| NFR-5 | Fault Tolerance | System must degrade safely under failures | Safe fallback mode |
| NFR-6 | Communication Delay Tolerance | System must tolerate V2X delays | ≤ 100 ms delay |
| NFR-7 | Computational Efficiency | Scheduling must remain bounded | O(n²) or better |
| NFR-8 | Embedded Feasibility | Must run on embedded hardware | CPU + memory bounded |
| NFR-9 | Collision Avoidance | No two vehicles shall occupy the same conflict area or collision block simultaneously. | Strict mutual exclusion of conflict zones |
| NFR-10 | Cross-Collision Point Safety | Vehicles from conflicting directions shall not reach the same collision point at the same time. | Conflict-free scheduling |
| NFR-11 | Permission Before Entry | Vehicles shall only enter the intersection after receiving crossing permission from the intersection coordination unit. | Entry only with valid authorization |
| NFR-12 | Minimum Vehicle Separation | Vehicles traveling in the same lane shall maintain a safe separation distance. | Separation distance ≥ safety threshold |
| NFR-13 | Safe Turning Speed | Vehicles performing left or right turns shall reduce speed to safe turning limits. | Turning speed ≤ configured safety limit |
| NFR-14 | Communication Failure Safety | A disconnected vehicle without valid permission shall stop before the stop line. | Fail-safe stop on communication loss |
| NFR-15 | Sensor Failure Safe Mode | If roadside or vehicle sensors become unreliable, the system shall switch to a safe operational mode. | Mandatory safe-mode activation |
| NFR-16 | Emergency Vehicle Priority Safety | Emergency vehicles shall receive priority crossing while maintaining safety for all other road users. | Emergency traffic receives highest priority |
| NFR-17 | Controller Response Time | The intersection coordination unit shall compute and send crossing permission within 2 seconds after receiving vehicle data. | Response time ≤ 2 s |
| NFR-18 | Maximum Vehicle Waiting Time | No vehicle shall wait indefinitely for crossing permission during normal operation. | Bounded waiting time |
| NFR-19 | Safe Intersection Clearance Time | Vehicles shall clear reserved conflict areas within the assigned crossing time window. | Clearance within allocated time slot |
| NFR-20 | Communication Timeout Detection | The system shall detect V2I or V2X communication failure within 3 seconds. | Failure detection ≤ 3 s |
| NFR-21 | Emergency Reaction Time | The controller shall cancel unsafe permissions immediately after detecting an obstacle, accident, or emergency vehicle. | Immediate emergency response |
| NFR-22 | Sensor Update Timing | Vehicle sensors, roadside cameras, and radars shall provide periodic environment updates in real time. | Continuous real-time updates |
| NFR-23 | Safe Switching Delay | The controller shall maintain a safe temporal separation between conflicting vehicle movements. | Minimum switching delay enforced |
| NFR-24 | Congestion Scheduling Time | The controller shall continuously recompute crossing schedules during high traffic density conditions. | Dynamic schedule recomputation within operational deadline |

## 2.7.2 Non-Functional Requirements (NFR)

| ID | Requirement | Description | Mapped Block(s) | Verification |
| --- | --- | --- | --- | --- |
| NFR-1 | Real-Time Responsiveness | ≤ 50 ms response | Scheduling Engine, Vehicle Interface | WCET + latency profiling |
| NFR-2 | Determinism | Same input → same output | Global State Manager, Verification Module | Replay-based validation |
| NFR-3 | Scalability | Support 8 lanes | Lane State Builder, Scheduler | Load testing |
| NFR-4 | Availability | ≥ 99.9% uptime | Entire system | Fault injection testing |
| NFR-5 | Fault Tolerance | Safe degradation | Global State Manager, Verification Module | Failure mode analysis |
| NFR-6 | Communication Delay Tolerance | ≤ 100 ms delay tolerance | Vehicle Interface | Network latency simulation |
| NFR-7 | Computational Efficiency | O(n²) or better | Scheduling Engine | Complexity analysis |
| NFR-8 | Embedded Feasibility | Run on embedded hardware | Entire control core | Resource profiling |
| NFR-9 | Collision Freedom | No conflicting vehicles simultaneously | Conflict Detection, Verification | Model checking (UPPAAL-ready) |
| NFR-10 | Safe Separation | Maintain safe distance | Conflict Detection, Scheduling | Constraint validation |
| NFR-11 | Deadlock Freedom | No circular wait | Arbitration, Scheduling, Verification | Graph-based + formal analysis |
| NFR-12 | Bounded Decision Time | ≤ 10–50 ms scheduling WCET | Scheduling Engine | Worst-case execution analysis |
| NFR-13 | Maximum Waiting Time | Bounded delay per lane | Scheduling, Reservation Manager | Queue analysis |
| NFR-14 | Atomic Scheduling | No partial commits | Reservation Manager | Transaction-like commit validation |
| NFR-15 | Emergency Preemption | Emergency vehicle priority | Arbitration Manager | Priority override test |
| NFR-16 | Temporal Consistency | No stale execution | Global State Manager, Vehicle Interface | Time synchronization validation |

## 2.8 Concept Selection and Evaluation

Before selecting the final smart intersection architecture, several alternative intersection management approaches were evaluated.

## Pugh Matrix Reference Concept: Traditional Traffic Lights (Solution A)

Scoring: "+" = Better than reference, "0" = Same as reference, "-" = Worse than reference

| Criteria | Weight | Traffic Lights (Ref) | Roundabout | Reservation-Based | Distributed Negotiation |
| --- | --- | --- | --- | --- | --- |
| Safety | 5 | 0 | + | + | - |
| Traffic Throughput | 5 | 0 | + | + | + |
| Waiting Time Reduction | 5 | 0 | + | + | + |
| Real-Time Suitability | 4 | 0 | 0 | + | - |
| Embedded Implementation | 4 | 0 | + | + | - |
| Formal Verification | 5 | 0 | - | + | - |
| Scalability | 3 | 0 | 0 | + | + |
| Complexity | 3 | 0 | + | - | - |
| V2X Integration | 4 | 0 | - | + | + |
| Autonomous Vehicle Compatibility | 5 | 0 | 0 | + | + |

## Weighted Evaluation Matrix

Using numerical scores:

| Criteria | Weight | Roundabout | Reservation-Based | Distributed |
| --- | --- | --- | --- | --- |
| Safety | 5 | 5 | 5 | -5 |
| Throughput | 5 | 5 | 5 | 5 |
| Waiting Time | 5 | 5 | 5 | 5 |
| Real-Time Suitability | 4 | 0 | 4 | -4 |
| Embedded Feasibility | 4 | 4 | 4 | -4 |
| Formal Verification | 5 | -5 | 5 | -5 |
| Scalability | 3 | 0 | 3 | 3 |
| Complexity | 3 | 3 | -3 | -3 |
| V2X Integration | 4 | -4 | 4 | 4 |
| AV Compatibility | 5 | 0 | 5 | 5 |
| Total | — | 13 | 37 | 1 |

## Selected Solution

The Reservation-Based Intersection Management System achieved the highest overall score (37 points) and was therefore selected as the preferred system concept.

The approach offers:

- Collision-free scheduling

- Real-time decision making

- High traffic throughput

- Compatibility with autonomous vehicles

- Suitability for FreeRTOS implementation

- Strong support for formal verification using UPPAAL

This makes it the most appropriate solution for the objectives and requirements defined in this projec

# Chapter 3 – System Modelling and Architecture

## 3.1 High-Level Architecture

Figure 1: Block Diagram

The Block Definition Diagram (BDD) for the smart intersection system defines the structural decomposition of the controller into well-scoped functional blocks rather than a single monolithic unit. At the highest level, the system is partitioned into perception, decision, execution, and state management layers. The perception layer includes the Sensor Subsystem and Lane State Builder, which convert raw V2X and sensor data into a structured traffic representation. The decision layer is further decomposed into Conflict Detection, Arbitration, Scheduling, and Verification, each responsible for a distinct transformation of system knowledge from safety analysis to time-space planning. The execution layer contains the Reservation Manager and Vehicle Interface, which ensure that validated schedules are safely committed and physically executed. The Global State Manager acts as a shared system memory, maintaining a consistent and synchronized view of all critical system states across these layers.

This BDD structure also explicitly captures the relationships and dependencies between blocks, showing how outputs of one subsystem become inputs or shared state updates for others. Instead of embedding all logic inside a single controller, each block has a clearly defined responsibility and interface. This makes the system behaviour traceable from requirements to implementation, since each block can be directly mapped to specific FR, NFR, and STR requirements. It also enables hierarchical reasoning, where complex behaviour emerges from the interaction of simpler, verifiable components rather than from an opaque centralized algorithm.

Compared to a monolithic controller design, this decomposition provides significantly better modularity and maintainability. In a monolithic system, perception, safety reasoning, scheduling, and execution logic are tightly coupled, making it difficult to isolate faults, update individual functions, or formally verify correctness. Any modification to one part of the controller risks unintended side effects elsewhere, which becomes critical in safety-sensitive systems like autonomous intersections. In contrast, the BDD-based architecture isolates concern into independent blocks with well-defined interfaces, reducing coupling and improving system robustness under changes or extensions.

Furthermore, the distributed architecture is better suited for real-time and safety-critical verification. Each subsystem can be individually analysed for timing constraints, correctness, and resource usage, enabling formal methods such as UPPAAL to be applied more effectively. It also allows direct mapping to RTOS task structures, improving schedulability analysis and implementation feasibility on embedded platforms. Overall, while monolithic controllers may be simpler initially, the BDD-based modular architecture provides stronger guarantees in terms of safety, scalability, verifiability, and requirement traceability, which are essential for intelligent intersection systems.

## 3.2 Activity Modelling

### Activity Diagram

### Behavioural Analysis

## 3.3 State Machine Modelling

## The State Machine Diagram represents the behaviour of the Reservation-Based Intersection Management Controller. The controller begins in the Idle state and waits for vehicle requests. When a vehicle arrives, its position, speed, and intended movement are collected and validated using sensor and V2X data.

### Controller States

### Based on the received information, the controller classifies the request according to the operational scenarios. During normal operation (S1), vehicles are processed using First-Come-First-Served (FCFS) reservation scheduling. For non-conflicting movements (S2), compatible vehicles may receive overlapping reservation slots and cross simultaneously. For conflicting movements (S3), an arbitration process assigns separate time slots to ensure collision-free crossing. Emergency vehicle requests (S4) activate the Emergency Override state, which temporarily prioritizes emergency traffic over normal traffic. If sensor or V2X data becomes unreliable (S5), the controller enters Safe Mode and issues stop or wait commands until reliable information is restored.

### Transition Analysis

### Before granting permission, all reservations are verified to ensure that no collision or conflict can occur. Once permission is granted, the controller monitors the vehicle while it crosses the intersection. After the vehicle exits the reserved conflict zone, the reservation is released and the controller returns to the Idle state, ready to process new requests.

## 3.7 Sequence Modelling

### Vehicle Processing Sequence

Figure 2: Sequence Diagram

The Vehicle Processing Sequence describes the normal operation of the smart intersection controller from vehicle arrival to movement execution. The sequence begins when vehicle state and intent information are received through sensors and V2X communication. The Sensor Subsystem collects the raw data and forwards it to the Lane State Builder, which constructs a structured LaneState representing the current traffic situation. This information is stored in the Global State Manager and passed to the Conflict Detection Engine, where potential conflicts between vehicle trajectories are identified. If conflicts exist, the Arbitration Manager applies priority and safety rules to generate a SafeConstraintSet; otherwise, default constraints are used.

Using the available constraints, the Scheduling Engine generates a SchedulePlan that defines the temporal order and reservation of intersection resources. The Verification Module then validates the generated schedule against safety, timing, and consistency requirements. If the schedule is invalid, a replanning process is triggered until a valid schedule is produced. Once approved, the Reservation Manager commits the schedule and creates active reservations. The Vehicle Interface subsequently issues movement commands to the vehicles and receives execution feedback. Finally, the execution results are stored in the Global State Manager, completing the control cycle and providing an updated system state for the next scheduling iteration.

### Emergency Handling Sequence

The Emergency Handling Sequence is activated whenever an emergency vehicle is detected within the intersection environment. During normal operation, emergency vehicle information may be received through V2X messages or dedicated priority signals. When such a request is detected, the Vehicle Interface forwards the emergency event to the Arbitration Manager. The Arbitration Manager immediately applies priority override rules, ensuring that emergency traffic receives precedence over regular vehicles while maintaining safety constraints.

Following the priority update, a new set of scheduling constraints is generated and forwarded to the Scheduling Engine. The scheduler computes an updated priority schedule that incorporates the emergency vehicle's route and timing requirements. This schedule is then validated by the Verification Module to ensure that collision freedom, safe separation, and timing constraints remain satisfied despite the priority override. After successful verification, the Reservation Manager commits the emergency schedule and the Vehicle Interface transmits the corresponding movement commands. The resulting execution feedback and updated priority state are recorded in the Global State Manager, ensuring that subsequent control cycles operate using the latest emergency-aware system state. This process allows emergency vehicles to receive immediate priority while preserving safe and deterministic intersection operation.

# Chapter 4 – Traffic Optimization Strategies

## Design Rationale and Trade-Off Analysis

### 1. System Objective and Problem Class

The intersection management system addresses a dynamic, safety-critical scheduling problem in which vehicles with heterogeneous movement intents must be coordinated over shared spatial resources under strict real-time constraints. The problem can be formally interpreted as a dynamic, weighted, conflict-constrained resource allocation problem with hard safety requirements and bounded execution deadlines. The primary objective is not global performance optimization, but the construction of a feasible, conflict-free, and temporally bounded schedule that satisfies all safety constraints while remaining computationally tractable for real-time embedded execution and formal verification. This places the problem in a class where exact optimal scheduling (e.g., maximum-weight independent set over a dynamic conflict graph with temporal constraints) is computationally intractable in general cases and incompatible with model-checking requirements.

### 2. Design Rationale of the Greedy Scheduling Approach

The adopted scheduling strategy is based on a greedy, priority-weighted selection mechanism augmented with aging-based fairness and cost-aware biasing. At each scheduling cycle, each vehicle request is assigned a dynamic priority score:

Scorei= waitTimei- costi

The scheduler selects the highest scoring feasible request subject to safety constraints derived from spatial conflict conditions. This design is motivated by three primary engineering principles:

| Design Aspect | Rationale | Effect / Guarantee |
| --- | --- | --- |
| Real-Time Feasibility | Scheduling uses a greedy evaluation strategy to avoid exponential or combinatorial optimization. Decisions are computed with bounded complexity suitable for embedded execution. | Guarantees bounded computation time and compliance with strict real-time constraints (NFR-1, NFR-7, NFR-8). |
| Deterministic Behavior for Verification | The scheduling rule is fully deterministic given system state, including fixed tie-breaking. This supports reproducible execution traces in timed automata models. | Ensures identical outputs for identical inputs, enabling formal verification and tractable state-space exploration (NFR-2). |
| Incremental Fairness via Aging | A monotonic aging mechanism increases priority of waiting requests over time, counteracting starvation in pure priority schemes. | Provides weak fairness: all continuously pending requests are eventually served under stable conditions (NFR-18). |

### 3. Safety Model and Separation of Concerns

Safety is enforced not through the scoring function, but through an explicit conflict-aware execution layer (intersection controller). Spatial constraints are modeled as mutually exclusive resource regions (quadrants or conflict zones), and vehicle execution is permitted only if no overlap exists with already reserved regions.

This design follows a strict separation:

- Scheduler layer: decides ordering and selection

- Controller layer: enforces safety constraints and resource exclusivity

This separation ensures that even in the presence of suboptimal or heuristic scheduling decisions, safety invariants remain structurally enforced.

### 4. Theoretical Properties and Guarantees

The proposed approach guarantees the following system-level properties under correct implementation:

| Property | Formal Statement | Guarantee / Interpretation |
| --- | --- | --- |
| Conflict Safety (Hard Guarantee) | No two concurrently executing vehicles are assigned overlapping spatial conflict zones. This corresponds to an invariant over a conflict graph ensuring independent set selection at execution time. | Ensures strict mutual exclusion in shared intersection regions. Safety is enforced as an invariant, preventing simultaneous occupation of conflicting zones. |
| Determinism | Given identical system states, the scheduler produces identical outputs. Determinism is achieved through a fixed scoring function and deterministic tie-breaking. | Guarantees reproducibility of scheduling decisions, enabling formal verification and consistent system behavior across executions. |
| Bounded Execution Time | Scheduling decisions are computed in polynomial time with respect to the number of active vehicles, ensuring compliance with strict real-time constraints. | Ensures predictable worst-case computation time and satisfies real-time system requirements. |
| Starvation Freedom (Weak Fairness) | Due to monotonic aging, any continuously pending request will eventually reach sufficient priority for selection, ensuring eventual service under fairness assumptions. | Guarantees eventual service of all persistent requests, preventing indefinite postponement under sustained load. |

### 5. Trade-Off Analysis

| Trade-Off Dimension | Scientific Statement | Impact / Interpretation |
| --- | --- | --- |
| Optimality vs. Real-Time Feasibility | The greedy approach does not guarantee global optimality with respect to system-wide metrics (e.g., total waiting time, throughput, or long-horizon congestion minimization). This is due to its myopic decision-making, which considers only the current system state without lookahead. Achieving global optimality would require solving a dynamic combinatorial optimization problem equivalent to repeated weighted independent set selection with temporal coupling, which is computationally intractable under real-time constraints. | Prioritizes bounded computation and embedded feasibility over global performance optimality. Ensures real-time operation at the cost of long-horizon optimal scheduling. |
| Structural Safety vs. Optimization Integration | Safety constraints are decoupled from the scoring function and enforced at the execution layer. While this guarantees strong correctness properties, it prevents joint optimization of safety and performance within a unified objective function. This separation reduces model complexity and improves verifiability. | Improves formal verification and safety guarantees, but limits global resource optimization due to decoupled decision layers. |
| Determinism vs. Flexibility | The scheduler is deterministic, improving reproducibility and formal verifiability. However, this reduces flexibility in resolving equivalent priority cases and may introduce structural bias based on evaluation order. | Ensures predictable and verifiable behavior, but sacrifices adaptive or stochastic optimization strategies and introduces potential deterministic bias. |
| Local Optimality vs. Global Optimality | Each scheduling decision is locally optimal with respect to the instantaneous scoring function. However, the absence of future-state evaluation prevents global optimality over time horizons. This limitation is inherent to greedy scheduling in dynamic systems but is mitigated through aging and conflict-aware execution constraints. | Ensures efficient per-step decisions, but does not guarantee globally optimal system-wide performance over time. |

### 6. Justification of Design Choice

The selection of a greedy, aging-based scheduling strategy is selected for the following system constraints:

- Strict real-time deadlines requiring bounded computation

- Embedded execution constraints limiting algorithmic complexity

- Requirement for deterministic behavior under formal verification

- Hard safety constraints requiring explicit conflict avoidance

- Dynamic and partially observable system state

Within this context, greedy scheduling represents a pragmatic optimality under constraints, where the objective shifts from maximizing performance to guaranteeing correctness, safety, and feasibility. The presented scheduling approach is a deterministic, real-time feasible, conflict-aware greedy heuristic with aging-based fairness, designed specifically for safety-critical intersection management under formal verification constraints. While it does not achieve global optimality in classical optimization terms, it provides strong guarantees in safety, bounded execution, and determinism, which are primary requirements in embedded autonomous coordination systems.

# Chapter 5 – Formal Verification Using UPPAAL(Nader)

## 6.1 Verification Objectives

## 6.2 System Abstraction

## 6.3 Timed Automata Design

## 6.4 Safety Verification

### Collision Freedom

### Mutual Exclusion

## 6.5 Liveness Verification

### Eventual Access

### Fairness

## 6.6 Timing Verification

### Waiting Time Bounds

### Response Time Constraints

## 6.7 Deadlock Analysis

## 6.8 Verification Results

# Chapter 6 – Real-Time Embedded System Design(Hassan)

## 7.1 Real-Time Requirements

## 7.2 FreeRTOS Architecture

## 7.3 Task Design

### Sensor Task

### Scheduler Task

### Arbitration Task

### Emergency Task

### Monitoring Task

## 7.4 Scheduling Strategy

### Fixed Priority

### RMS

### EDF

### Selected Scheduler

## 7.5 Inter-Task Communication

## 7.6 Synchronization Design

### Queues

### Semaphores

### Mutexes

## 7.7 WCET Analysis

## 7.8 Latency Analysis

## 7.9 Jitter Analysis

# Chapter 7 – HW/SW Co-Design (Sohan+Hassan)

## 8.1 Co-Design Objectives

## 8.2 Hardware–Software Partitioning

## 8.3 Candidate Hardware Accelerators

### Traffic Arbitration

### Sensor Preprocessing

### Timing Logic

## 8.4 FPGA-Based Architecture

## 8.5 ModelSim Validation

## 8.6 Trade-Off Analysis

# Chapter 8 – System Implementation(Umer)

## 9.1 Software Structure

## 9.2 Implemented Modules

## 9.3 Controller Logic

## 9.4 Communication Mechanisms

## 9.5 Integration Process

## 9.6 Implementation Challenges

# Chapter 9 – Testing and Validation (Umer)

## 10.1 Validation Strategy

## 10.2 Functional Testing

## 10.3 Congestion Testing

## 10.4 Emergency Vehicle Testing

## 10.5 Stress Testing

## 10.6 Real-Time Performance Testing

## 10.7 Verification Matrix

## 10.8 Test Results

# Chapter 11 – Results and Evaluation (Umer)

## 11.1 Throughput Analysis

## 11.2 Waiting Time Analysis

## 11.3 Queue Length Analysis

## 11.4 CPU Utilization Analysis

## 11.5 Deadline Miss Analysis

## 11.6 Jitter Analysis

## 11.7 Fairness Evaluation

## 11.8 Discussion of Results

# Chapter 12 – Conclusion and Future Work

## 12.1 Summary of Achievements

This project presented the design and implementation of a real-time autonomous intersection control system based on a hardware/software co-design architecture.

The main achievements include:

- Development of a FreeRTOS-based ARM controller for real-time vehicle request handling and scheduling.

- Implementation of a reservation-based FPGA accelerator for deterministic conflict detection and quadrant management.

- Design of a memory-mapped AXI-lite interface enabling structured communication between software and hardware.

- Development of a queue-based scheduling mechanism for handling dynamic vehicle arrivals.

- Integration of an emergency handling mechanism ensuring system override capability under critical conditions.

- Establishment of a predictable and bounded execution model using FPGA-controlled slot timing.

Overall, the system successfully demonstrates a scalable and deterministic approach to autonomous intersection management.

## 12.2 Research Questions (Tabular Summary)

| Research Question | Answer |
| --- | --- |
| RQ1: How can real-time vehicle coordination be achieved in a shared intersection environment? | Real-time coordination is achieved using a FreeRTOS-based ARM controller that manages incoming vehicle requests through a queueing mechanism, combined with an FPGA-based reservation system that ensures only non-conflicting vehicles are granted access to the intersection. |
| RQ2: How can hardware/software co-design improve scheduling efficiency and safety? | The system improves efficiency and safety by separating responsibilities: the ARM handles dynamic scheduling and queue management, while the FPGA performs deterministic conflict detection and reservation enforcement, reducing CPU load and ensuring predictable execution. |
| RQ3: How is deterministic and conflict-free execution guaranteed? | Deterministic execution is ensured through fixed quadrant mapping (Q1–Q4), reservation-based access control in FPGA, and bounded execution slots (SLOT_TIME), guaranteeing that no conflicting vehicles are executed simultaneously and all operations remain time-bounded. |

## 12.3 Limitations and Discussion

Despite its effectiveness in demonstrating a real-time hardware/software co-design approach for autonomous intersection control, the system has several inherent limitations. The current scheduling strategy is primarily FIFO-based, which ensures fairness but does not optimize global traffic throughput under high-density conditions. In addition, the FPGA design is based on a fixed intersection topology with predefined quadrant mapping, which limits scalability to more complex or irregular road networks. The AXI-lite communication interface supports only single-request transactions at a time, restricting parallel request handling and potentially reducing efficiency under heavy traffic load. Furthermore, the system does not yet include learning-based or adaptive optimization mechanisms, meaning it cannot dynamically improve scheduling decisions based on historical traffic patterns. Finally, the traffic model assumes simplified and idealized vehicle arrival behavior, which may not fully reflect the variability and burstiness of real urban traffic environments.

## 12.4 Future Enhancements

Future improvements can significantly extend the capabilities of the system. Advanced scheduling algorithms such as priority-based or cost-based approaches can be introduced, along with adaptive scheduling driven by real-time traffic density estimation. The FPGA architecture can be further optimized through parallel reservation processing and more efficient conflict detection logic to increase throughput. On the communication side, upgrading from AXI-lite to AXI-stream or DMA-based interfaces would enable higher bandwidth and support multiple simultaneous requests. Additionally, integrating AI-based traffic prediction models could allow proactive scheduling decisions based on expected vehicle arrivals. Finally, the system can be scaled from a single intersection to a coordinated multi-intersection network, enabling distributed and intelligent urban traffic management.

This project demonstrates a successful hardware/software co-design methodology for autonomous intersection control, combining deterministic FPGA-based safety enforcement with a flexible FreeRTOS-based scheduling system. The architecture provides a foundation for future scalable, intelligent, and real-time traffic management systems.

# References

[1] N. Carpinelli, Computer Systems Organization and Architecture, Wiley, 2018.

[2] A. Burns and A. Wellings, Real-Time Systems and Programming Languages, Addison-Wesley, 2009.

[3] FreeRTOS, “FreeRTOS Documentation,” https://www.freertos.org

[4] ARM Ltd., AMBA AXI and ACE Protocol Specification, ARM Technical Reference Manual, 2010–2020.

[5] Xilinx Inc., AXI Reference Guide (UG1037 / AXI Protocol Overview), Xilinx Documentation.

[6] IEEE Standard Association, IEEE 1076-2008: VHDL Language Reference Manual.

[7] D. E. Comer, Computer Networks and Internets, Pearson, 2018.

[8] L. Kleinrock, Queueing Systems, Volume 1: Theory, Wiley-Interscience, 1975.

[9] M. P. Thadakamalla et al., “Reservation-Based Intersection Management for Autonomous Vehicles,” IEEE Intelligent Transportation Systems (ITS) literature.

[10] J. Axelsson, “Safety in Vehicle Automation Systems,” IEEE Transactions on Intelligent Vehicles, 2017.

[11] S. Yi, Z. Hao, “FPGA-Based Real-Time Traffic Control Systems,” IEEE Embedded Systems Letters, 2019.

[12] Xilinx Inc., Zynq-7000 SoC Technical Reference Manual, Xilinx Documentation.

# Appendices

## Appendix A – Complete Requirements Specification

## Appendix B – SysML Diagrams

## Appendix C – UPPAAL Models

## Appendix D – FreeRTOS Configuration

## Appendix E – Additional Simulation Results

## Appendix F – Source Code Excerpts