# PHASE 1.2 — CONSOLIDATED REQUIREMENTS SPECIFICATION

# 1. ENGINEERING INTERPRETATION

This document defines the complete requirements for a real-time Intersection Control Unit (ICU) managing a single 4-way multi-lane intersection (2 lanes per direction). The system operates as a: real-time, safety-critical, multi-agent scheduling system over a shared intersection conflict zone.

It is structured into three requirement classes:

- Functional Requirements (system behaviour)

- Non-Functional Requirements (performance constraints)

- Safety & Timing Requirements (hard real-time invariants)

# 2. ASSUMPTIONS

- Single 4-way intersection (N/E/S/W)

- 2 lanes per direction (8 incoming lanes total)

- No traffic lights

- No pedestrians

- Vehicles are cooperative or partially cooperative (V2X assumed)

- ICU is the sole decision authority

- Vehicles provide: position, velocity, intended maneuver (left / straight / right)

# 3. REQUIREMENTS SPECIFICATION

# 3.1 FUNCTIONAL REQUIREMENTS (WHAT THE SYSTEM DOES)

| ID | Requirement | Description | Input | Output |
| --- | --- | --- | --- | --- |
| FR-1 | Vehicle State Acquisition | System shall collect real-time vehicle state data | V2X, sensors | Structured lane-wise vehicle state |
| FR-2 | Lane Monitoring | System shall maintain lane occupancy and queue state | Vehicle arrivals | Lane state model |
| FR-3 | Trajectory Interpretation | System shall map vehicle intent to intersection paths | Vehicle intent | Conflict zone path mapping |
| FR-4 | Conflict Detection | System shall detect spatial-temporal conflicts | Trajectories | Conflict graph |
| FR-5 | Time-Space Reservation | System shall assign non-overlapping intersection slots | Conflict graph | Schedule plan |
| FR-6 | Movement Authorization | System shall issue movement permissions | Schedule | Vehicle commands |
| FR-7 | Scheduling Update | System shall update schedule in real time | New arrivals | Updated schedule |
| FR-8 | State Maintenance | System shall maintain global intersection state | All inputs | System state model |
| FR-9 | Deadlock Prevention (Functional) | System shall avoid full intersection blocking | Lane states | Safe schedule |
| FR-10 | Emergency Override Handling | System shall prioritize emergency vehicles | Emergency input | Preempted schedule |

# 3.2 NON-FUNCTIONAL REQUIREMENTS (HOW WELL IT PERFORMS)

| ID | Requirement | Description | Constraint |
| --- | --- | --- | --- |
| NFR-1 | Real-Time Responsiveness | System must respond to events within bounded time | ≤ 50 ms latency |
| NFR-2 | Determinism | Same input must produce identical output | 100% deterministic |
| NFR-3 | Scalability | System must handle all lanes under peak load | 8 lanes active |
| NFR-4 | Availability | System must operate continuously | ≥ 99.9% uptime |
| NFR-5 | Fault Tolerance | System must degrade safely under failures | Safe fallback mode |
| NFR-6 | Communication Delay Tolerance | System must tolerate V2X delays | ≤ 100 ms delay |
| NFR-7 | Computational Efficiency | Scheduling must remain bounded | O(n²) or better |
| NFR-8 | Embedded Feasibility | Must run on embedded hardware | CPU + memory bounded |

# 3.3 SAFETY & TIMING REQUIREMENTS (HARD CONSTRAINTS)

| ID | Requirement | Description | Constraint |
| --- | --- | --- | --- |
| STR-1 | Collision Freedom | No two conflicting vehicles occupy intersection simultaneously | Strict mutual exclusion |
| STR-2 | Safe Separation | Vehicles maintain safe spacing in conflict zone | ≥ safety threshold |
| STR-3 | Deadlock Freedom | System must avoid full intersection blockage | No circular wait |
| STR-4 | Bounded Decision Time | Scheduling must complete within strict deadline | ≤ 10–50 ms WCET |
| STR-5 | Maximum Waiting Time | Vehicle delay must be bounded per lane | Bounded latency |
| STR-6 | Atomic Scheduling | Scheduling must be indivisible | No partial commits |
| STR-7 | Emergency Preemption | Emergency vehicles override all scheduling | Immediate priority |
| STR-8 | Temporal Consistency | System state must always be up-to-date | No stale execution |

# 4. SYSTEM OUTCOMES

This requirements model defines UPPAAL readiness:

- STR-1 → safety invariant (no collision state)

- STR-3 → deadlock freedom property

- FR-4 → conflict detection automaton

- FR-5 → resource allocation model

### FreeRTOS readiness:

- FR-1 → sensor task

- FR-4 → conflict detection task

- FR-5 → scheduler task

- NFR-1 → task deadlines

### Queueing theory readiness:

- lane arrivals = λ

- ICU processing = μ

- congestion = ρ = λ/μ

## 4.3 System Boundary Definition

- Input: vehicles + sensors + emergency signals

- Core: ICU scheduling engine

- Output: movement permissions / reservations

# 5. EDGE CASES & RISKS

- Lane starvation under heavy directional load

- Sensor/V2X inconsistency causing wrong state estimation

- Deadlock under symmetric traffic arrival

- Conflict explosion under high density traffic

- Emergency override disrupting fairness

- Timing violations under peak load

# 6. VERIFICATION STRATEGY

- Requirement traceability: FR → ICU functions, NFR → system performance tests STR → formal verification properties

- Simulation: high-density traffic stress test, random arrival patterns

- Formal methods:  UPPAAL model checking (safety + deadlock freedom)

- Timing validation: WCET analysis (real-time constraint validation)