Autonomous Traffic-Lightless Intersection Control

1.Safety requirements

The safety requirements define the conditions necessary to ensure safe operation of the autonomous traffic lightless intersection. They focus on preventing collisions, maintaining safe vehicle separation, controlling vehicle entry through authorization, handling communication and sensor failures safely, and ensuring priority handling for emergency vehicles while protecting all road users.

| ID | Requirement | Description | Constraint |
| --- | --- | --- | --- |
| SR1 | Collision Avoidance | No two vehicles shall occupy the same conflict area or collision block simultaneously. | Strict mutual exclusion of conflict zones |
| SR2 | Cross-Collision Point Safety | Vehicles from conflicting directions shall not reach the same collision point at the same time. | Conflict-free scheduling |
| SR3 | Permission Before Entry | Vehicles shall only enter the intersection after receiving crossing permission from the intersection coordination unit. | Entry only with valid authorization |
| SR4 | Minimum Vehicle Separation | Vehicles traveling in the same lane shall maintain a safe separation distance. | Separation distance ≥ safety threshold |
| SR5 | Safe Turning Speed | Vehicles performing left or right turns shall reduce speed to safe turning limits. | Turning speed ≤ configured safety limit |
| SR6 | Communication Failure Safety | A disconnected vehicle without valid permission shall stop before the stop line. | Fail-safe stop on communication loss |
| SR7 | Sensor Failure Safe Mode | If roadside or vehicle sensors become unreliable, the system shall switch to a safe operational mode. | Mandatory safe-mode activation |
| SR8 | Emergency Vehicle Priority Safety | Emergency vehicles shall receive priority crossing while maintaining safety for all other road users. | Emergency traffic receives highest priority |

2. Timing requirements

The timing requirements specify the real-time performance constraints of the intersection control system. They ensure that vehicle requests, scheduling decisions, communication monitoring, emergency responses, and sensor updates are processed within defined time limits, guaranteeing efficient traffic flow, bounded waiting times, and safe coordination of vehicle movements under both normal and congested traffic conditions.

| ID | Requirement | Description | Constraint |
| --- | --- | --- | --- |
| TR1 | Controller Response Time | The intersection coordination unit shall compute and send crossing permission within 2 seconds after receiving vehicle data. | Response time ≤ 2 s |
| TR2 | Maximum Vehicle Waiting Time | No vehicle shall wait indefinitely for crossing permission during normal operation. | Bounded waiting time |
| TR3 | Safe Intersection Clearance Time | Vehicles shall clear reserved conflict areas within the assigned crossing time window. | Clearance within allocated time slot |
| TR4 | Communication Timeout Detection | The system shall detect V2I or V2X communication failure within 3 seconds. | Failure detection ≤ 3 s |
| TR5 | Emergency Reaction Time | The controller shall cancel unsafe permissions immediately after detecting an obstacle, accident, or emergency vehicle. | Immediate emergency response |
| TR6 | Sensor Update Timing | Vehicle sensors, roadside cameras, and radars shall provide periodic environment updates in real time. | Continuous real-time updates |
| TR7 | Safe Switching Delay | The controller shall maintain a safe temporal separation between conflicting vehicle movements. | Minimum switching delay enforced |
| TR8 | Congestion Scheduling Time | The controller shall continuously recompute crossing schedules during high traffic density conditions. | Dynamic schedule recomputation within operational deadline |